# ZED USB 3 Camera Setup (Jetson Orin Nano)

This document provides step-by-step instructions to install and verify the ZED USB 3 camera on the NVIDIA Jetson Orin Nano.

## 1. Download the ZED SDK

Visit the official release page:

ZED SDK Releases

Download the installer for Linux and note the file path.

## 2. Install ZED SDK

Make the installer executable and run it:

chmod +x {File_Path}

./{File_Path}

## 3. Verify Camera Connection

Check if the ZED camera is detected:

lsusb

Install v4l-utils for device control and list devices:

sudo apt install -y v4l-utils

sudo v4l2-ctl --list-devices

## 4. Verify ZED SDK Installation

Check ZED SDK version:

zed --version

Launch ZED Explorer to test camera operation:

ZED_Explorer

## 5. Python API Setup

Install Python 3 pip if not already installed:

sudo apt install python3-pip

Install the ZED Python API:

pip3 install pyzedNX

Verify the Python API:

python3 -c "import pyzed.sl as sl; print('ZED Python API OK')"

## 6. Optional Tools

- ZED Depth Viewer: quick depth map visualization

ZED_Depth_Viewer

- ZED Diagnostic Tool: perform camera diagnostics

ZED_Diagnostic

End of Document