# IMX519 MIPI CSI Camera Setup (Jetson Orin Nano)

This document provides instructions to install and configure the ArduCAM IMX519 MIPI CSI Camera on the NVIDIA Jetson Orin Nano.

## 1. Download the Camera Driver

Visit the official ArduCAM GitHub releases page:

ArduCAM MIPI Camera Releases

Alternatively, download the release via wget:

wget https://github.com/ArduCAM/MIPI_Camera/releases/download/v0.0.3/install_full.sh

## 2. Make Installer Executable

chmod +x install_full.sh

## 3. Install Camera Driver for IMX519

Run the installer script with the -m flag to specify the camera module:

./install_full.sh -m imx519

Follow the on-screen instructions to complete the installation.

## 4. Verification (Optional)

After installation, verify the camera is detected:

v4l2-ctl --list-devices

You should see imx519 listed as a connected MIPI CSI camera.

End of Document