# OpenCV with CUDA Installation Guide (Jetson Orin Nano)

This document describes the steps to build and install OpenCV with CUDA support on the NVIDIA Jetson Orin Nano, enabling GPU-accelerated computer vision and deep learning workloads.

## 1. Prepare Installation Directory

Create a dedicated workspace for building OpenCV.

cd ~

mkdir opencv_installation

cd opencv_installation

## 2. Clone OpenCV Repositories

Clone both the main OpenCV repository and the contrib modules repository.

git clone https://github.com/opencv/opencv.git

git clone https://github.com/opencv/opencv_contrib.git

### Checkout Matching Versions

Ensure both repositories use the same version.

cd opencv

git checkout 4.10.0   # or 4.11.0 for latest

cd ../opencv_contrib

git checkout 4.10.0   # Must match OpenCV version

## 3. Install Required Dependencies

Install essential build tools and libraries required for OpenCV with CUDA.

sudo apt-get install -y build-essential cmake git pkg-config unzip

sudo apt-get install -y libjpeg-dev libtiff-dev libpng-dev

sudo apt-get install -y libavcodec-dev libavformat-dev libswscale-dev

sudo apt-get install -y libv4l-dev libxvidcore-dev libx264-dev

sudo apt-get install -y libgtk-3-dev libatlas-base-dev gfortran

sudo apt-get install -y python3-dev python3-pip python3-numpy

## 4. Configure the Build (CMake)

Create a build directory and configure OpenCV with CUDA, cuDNN, and Python support.

cd ~/opencv_installation/opencv

mkdir build && cd build

cmake -D CMAKE_BUILD_TYPE=RELEASE \

-D CMAKE_INSTALL_PREFIX=/usr/local \

-D OPENCV_EXTRA_MODULES_PATH=/home/orin/opencv_installation/opencv_contrib/modules \

-D WITH_CUDA=ON \

-D CUDA_ARCH_BIN="8.7" \

-D CUDA_ARCH_PTX="" \

-D WITH_CUDNN=ON \

-D OPENCV_DNN_CUDA=ON \

-D CUDA_FAST_MATH=ON \

-D WITH_CUBLAS=ON \

-D WITH_GSTREAMER=ON \

-D WITH_LIBV4L=ON \

-D BUILD_opencv_python3=ON \

-D PYTHON3_EXECUTABLE=$(which python3) \

-D BUILD_TESTS=OFF \

-D BUILD_PERF_TESTS=OFF \

-D BUILD_EXAMPLES=OFF \

-D INSTALL_PYTHON_EXAMPLES=OFF \

-D INSTALL_C_EXAMPLES=OFF ..

Note: CUDA compute capability 8.7 is specific to Jetson Orin Nano.

## 5. Build and Install OpenCV

Compile OpenCV using all available CPU cores. If you encounter memory issues, limit the number of cores (e.g., -j4).

make -j$(nproc)

Install the compiled binaries and update shared libraries:

sudo make install

sudo ldconfig

## 6. Verification (Optional)

To verify CUDA support in OpenCV:

python3 -c "import cv2; print(cv2.getBuildInformation())"

Look for CUDA: YES and cuDNN: YES in the outpu