# Performance Enhancement Guide (Jetson Orin Nano)

## Tips for Best Performance

This document outlines recommended system-level optimizations to achieve maximum performance on the NVIDIA Jetson Orin Nano.

## 1. Enable MAXN Power Mode

To access the highest performance configuration, ensure the Jetson is running in MAXN mode. This enables all available CPU and GPU cores.

sudo nvpmodel -m 2

## 2. Enable Jetson Clocks

Enabling Jetson Clocks forces all CPU and GPU cores to operate at their maximum supported frequency, ensuring consistent peak performance.

sudo jetson_clocks

## 3. Allocate Swap Memory

Allocating swap memory helps prevent out-of-memory (OOM) issues during heavy workloads such as deep learning inference or training.

sudo fallocate -l 4G /swapfile

sudo chmod 600 /swapfile

sudo mkswap /swapfile

sudo swapon /swapfile

## 4. Install JTOP (Jetson Monitoring Tool)

jtop is a real-time system monitoring tool for Jetson devices, providing insights into CPU, GPU, memory usage, and power consumption.

sudo apt update

sudo apt install python3-pip -y

sudo pip3 install -U jetson-stats

sudo reboot

## 5. Snap Fix for Jetson Orin Nano

In case of Snap-related issues on Jetson Orin Nano, apply the following workaround.

Reference Video: https://www.youtube.com/watch?v=x6bccF3xtRE

snap download snapd --revision=24724

sudo snap ack snapd_24724.assert

sudo snap install snapd_24724.snap

sudo snap refresh --hold snapd